# GCP PostgreSQL 18 Patroni HA Lab Manual

Three Node High Availability Cluster Using Private GCP VMs

Windows gcloud administration  |  Ubuntu 24.04  |  PostgreSQL 18  |  Patroni  |  etcd

| Document purpose | Value |
| --- | --- |
| Audience | PostgreSQL DBAs, cloud engineers, DevOps engineers and trainers |
| Outcome | A working three-node Patroni cluster with automatic failover and controlled switchover |
| GCP project | postgresql-admin-lab-2026 |
| Primary region | asia-south1 |
| Security model | Private VMs, SSH through IAP, outbound access through Cloud NAT |

This manual replaces the earlier PDF commands that were truncated during conversion. Commands are separated by execution environment so Windows commands are not accidentally run inside Linux and vice versa.

# Lab Overview

The lab builds one PostgreSQL leader and two streaming replicas. Patroni manages initialization, leader election, promotion, demotion and replica recovery. A three-member etcd cluster stores the distributed cluster state. One etcd member is colocated with each PostgreSQL node for training efficiency.

| Node | Zone | Private IP | Installed services |
| --- | --- | --- | --- |
| pg-node-1 | asia-south1-a | 10.10.0.11 | PostgreSQL, Patroni, etcd |
| pg-node-2 | asia-south1-b | 10.10.0.12 | PostgreSQL, Patroni, etcd |
| pg-node-3 | asia-south1-c | 10.10.0.13 | PostgreSQL, Patroni, etcd |

## Ports

| Port | Purpose | Allowed source |
| --- | --- | --- |
| 22 | SSH through Identity Aware Proxy | 35.235.240.0/20 |
| 5432 | PostgreSQL client and replication traffic | 10.10.0.0/24 |
| 8008 | Patroni REST API | 10.10.0.0/24 |
| 2379 | etcd client traffic | 10.10.0.0/24 |
| 2380 | etcd peer traffic | 10.10.0.0/24 |

## Execution Environments

| Label | Run commands in |
| --- | --- |
| Windows CMD | Local Windows Command Prompt with Google Cloud CLI installed |
| Ubuntu all nodes | Run separately on pg-node-1, pg-node-2 and pg-node-3 |
| Ubuntu named node | Run only on the specified VM |

# Before You Start

The GCP project exists and billing is enabled.

Your Google account can create networks, routers, firewall rules and Compute Engine VMs.

The monthly budget alert is configured.

Google Cloud CLI is installed on Windows and gcloud auth login has completed.

The project is an isolated training environment. Do not reuse passwords shown in this manual for production.

# Phase 1 Verify the Local gcloud Configuration

## Step 1 Select and verify the project

gcloud config set project postgresql-admin-lab-2026
gcloud config get-value project

The second command must return postgresql-admin-lab-2026. Stop if it returns another project.

gcloud auth application-default set-quota-project postgresql-admin-lab-2026

## Step 2 Set the location

gcloud config set compute/region asia-south1
gcloud config set compute/zone asia-south1-a
gcloud config list

## Step 3 Confirm billing

gcloud billing projects describe postgresql-admin-lab-2026

Continue only when the output contains billingEnabled: true.

## Step 4 Enable APIs

gcloud services enable compute.googleapis.com iap.googleapis.com logging.googleapis.com monitoring.googleapis.com

# Phase 2 Build the Private Network

## Step 5 Create the VPC and subnet

gcloud compute networks create postgres-lab-vpc --subnet-mode=custom

gcloud compute networks subnets create postgres-lab-subnet --network=postgres-lab-vpc --region=asia-south1 --range=10.10.0.0/24 --enable-private-ip-google-access

## Step 6 Create firewall rules

gcloud compute firewall-rules create patroni-cluster-internal --network=postgres-lab-vpc --direction=INGRESS --action=ALLOW --rules=tcp:5432,tcp:8008,tcp:2379,tcp:2380,icmp --source-ranges=10.10.0.0/24 --target-tags=patroni-node

gcloud compute firewall-rules create patroni-iap-ssh --network=postgres-lab-vpc --direction=INGRESS --action=ALLOW --rules=tcp:22 --source-ranges=35.235.240.0/20 --target-tags=patroni-node

Never allow PostgreSQL, Patroni or etcd ports from 0.0.0.0/0.

## Step 7 Create Cloud NAT

The VMs have no public IP addresses. Cloud NAT supplies outbound internet access for Ubuntu, PGDG and PyPI downloads without permitting unsolicited inbound connections.

gcloud compute routers create postgres-lab-router --network=postgres-lab-vpc --region=asia-south1

gcloud compute routers nats create postgres-lab-nat --router=postgres-lab-router --region=asia-south1 --auto-allocate-nat-external-ips --nat-all-subnet-ip-ranges

gcloud compute routers nats describe postgres-lab-nat --router=postgres-lab-router --region=asia-south1

# Phase 3 Create Three Private VMs

Run each command as one line in Windows CMD. The VMs use fixed private IP addresses and no external IP address.

## Step 8 Create pg-node-1

gcloud compute instances create pg-node-1 --zone=asia-south1-a --machine-type=e2-standard-2 --network=postgres-lab-vpc --subnet=postgres-lab-subnet --private-network-ip=10.10.0.11 --no-address --tags=patroni-node --image-family=ubuntu-2404-lts-amd64 --image-project=ubuntu-os-cloud --boot-disk-size=30GB --boot-disk-type=pd-balanced

## Step 9 Create pg-node-2

gcloud compute instances create pg-node-2 --zone=asia-south1-b --machine-type=e2-standard-2 --network=postgres-lab-vpc --subnet=postgres-lab-subnet --private-network-ip=10.10.0.12 --no-address --tags=patroni-node --image-family=ubuntu-2404-lts-amd64 --image-project=ubuntu-os-cloud --boot-disk-size=30GB --boot-disk-type=pd-balanced

## Step 10 Create pg-node-3

gcloud compute instances create pg-node-3 --zone=asia-south1-c --machine-type=e2-standard-2 --network=postgres-lab-vpc --subnet=postgres-lab-subnet --private-network-ip=10.10.0.13 --no-address --tags=patroni-node --image-family=ubuntu-2404-lts-amd64 --image-project=ubuntu-os-cloud --boot-disk-size=30GB --boot-disk-type=pd-balanced

gcloud compute instances list

# Enable os login

gcloud auth list --filter=status:ACTIVE --format="value(account)"

gcloud projects add-iam-policy-binding postgresql-admin-lab-2026 --member="user:eswaribala70@gmail.com" --role="roles/compute.osAdminLogin"

Confirm the IAP role is also assigned:

gcloud projects add-iam-policy-binding postgresql-admin-lab-2026 --member="user:eswaribala70@gmail.com" --role="roles/iap.tunnelResourceAccessor"

Enable OS Login at project level

gcloud compute project-info add-metadata --project=postgresql-admin-lab-2026 --metadata=enable-oslogin=TRUE

Verify

gcloud compute project-info describe --project=postgresql-admin-lab-2026 --format="yaml(commonInstanceMetadata.items)"

gcloud projects get-iam-policy postgresql-admin-lab-2026 --flatten="bindings[].members" --filter="bindings.members:user:eswaribala70@gmail.com" --format="table(bindings.role)"

gcloud compute instances reset pg-node-1 --zone=asia-south1-a --project=postgresql-admin-lab-2026

gcloud compute instances describe pg-node-1 --zone=asia-south1-a --format="get(status)"

# Phase 4 Connect Through IAP

Use browser SSH.

Go to IAM instances open ssh

/*

Local ssh not needed

Open three Windows CMD windows and keep one session connected to each node.

gcloud compute ssh pg-node-1 --zone=asia-south1-a --tunnel-through-iap

gcloud compute ssh pg-node-2 --zone=asia-south1-b --tunnel-through-iap

gcloud compute ssh pg-node-3 --zone=asia-south1-c --tunnel-through-iap

If access is denied, the user requires roles/iap.tunnelResourceAccessor and suitable Compute Engine or OS Login permissions.

*/

## From node 1, test the other nodes:

## ping -c 3 10.10.0.12

## ping -c 3 10.10.0.13

## Step 11 Verify each VM

hostname
hostname -I
curl -I https://apt.postgresql.org

Expected private addresses are 10.10.0.11, 10.10.0.12 and 10.10.0.13. The curl test confirms outbound access through Cloud NAT.

# Phase 5 Install PostgreSQL 18 and Patroni

Run this phase on all three Ubuntu nodes.

## Step 12 Install prerequisites

sudo apt update
sudo apt install -y curl ca-certificates postgresql-common python3-venv python3-dev libpq-dev gcc etcd-server etcd-client

## Step 13 Install PostgreSQL 18 from PGDG

sudo /usr/share/postgresql-common/pgdg/apt.postgresql.org.sh
sudo apt update
sudo apt install -y postgresql-18 postgresql-client-18 postgresql-contrib-18
/usr/lib/postgresql/18/bin/postgres --version

## Step 14 Remove the package-created cluster

Patroni must initialize and own the cluster. This operation is valid only for the new empty 18/main cluster.

sudo pg_lsclusters
sudo pg_dropcluster --stop 18 main
sudo pg_lsclusters

## Step 15 Create directories

sudo install -d -o postgres -g postgres -m 700 /var/lib/postgresql/18/patroni
sudo install -d -o postgres -g postgres -m 2775 /var/run/postgresql

## Step 16 Install Patroni

sudo python3 -m venv /opt/patroni
sudo /opt/patroni/bin/pip install --upgrade pip
sudo /opt/patroni/bin/pip install "patroni[etcd3]" "psycopg[binary]"
/opt/patroni/bin/patroni --version
/opt/patroni/bin/patronictl version

# Phase 6 Configure the Three Member etcd Cluster

Patroni needs a Distributed Configuration Store (DCS). In this lab, etcd provides that service.

etcd does not store PostgreSQL table data. It stores small but critical HA coordination information, including:

- Which PostgreSQL node is currently the leader

- Patroni cluster configuration

- Leader-election lock

- Member status

- Failover and switchover state

- Initialization state

Run the stop command on all nodes before editing. Delete the data directory contents only because these are new lab nodes with no existing etcd data.

sudo systemctl stop etcd

sudo find /var/lib/etcd -mindepth 1 -delete

sudo find /var/lib/etcd -mindepth 1 -print

## Step 17 Configure pg-node-1

sudo nano /etc/default/etcd

ETCD_NAME="etcd1"
ETCD_DATA_DIR="/var/lib/etcd"
ETCD_LISTEN_PEER_URLS="http://10.10.0.11:2380"
ETCD_INITIAL_ADVERTISE_PEER_URLS="http://10.10.0.11:2380"
ETCD_LISTEN_CLIENT_URLS="http://127.0.0.1:2379,http://10.10.0.11:2379"
ETCD_ADVERTISE_CLIENT_URLS="http://10.10.0.11:2379"
ETCD_INITIAL_CLUSTER="etcd1=http://10.10.0.11:2380,etcd2=http://10.10.0.12:2380,etcd3=http://10.10.0.13:2380"
ETCD_INITIAL_CLUSTER_TOKEN="patroni-etcd-cluster"
ETCD_INITIAL_CLUSTER_STATE="new"

## Step 18 Configure pg-node-2

sudo nano /etc/default/etcd

ETCD_NAME="etcd2"
ETCD_DATA_DIR="/var/lib/etcd"
ETCD_LISTEN_PEER_URLS="http://10.10.0.12:2380"
ETCD_INITIAL_ADVERTISE_PEER_URLS="http://10.10.0.12:2380"
ETCD_LISTEN_CLIENT_URLS="http://127.0.0.1:2379,http://10.10.0.12:2379"
ETCD_ADVERTISE_CLIENT_URLS="http://10.10.0.12:2379"
ETCD_INITIAL_CLUSTER="etcd1=http://10.10.0.11:2380,etcd2=http://10.10.0.12:2380,etcd3=http://10.10.0.13:2380"
ETCD_INITIAL_CLUSTER_TOKEN="patroni-etcd-cluster"
ETCD_INITIAL_CLUSTER_STATE="new"

## Step 19 Configure pg-node-3

sudo nano /etc/default/etcd

ETCD_NAME="etcd3"
ETCD_DATA_DIR="/var/lib/etcd"
ETCD_LISTEN_PEER_URLS="http://10.10.0.13:2380"
ETCD_INITIAL_ADVERTISE_PEER_URLS="http://10.10.0.13:2380"
ETCD_LISTEN_CLIENT_URLS="http://127.0.0.1:2379,http://10.10.0.13:2379"
ETCD_ADVERTISE_CLIENT_URLS="http://10.10.0.13:2379"
ETCD_INITIAL_CLUSTER="etcd1=http://10.10.0.11:2380,etcd2=http://10.10.0.12:2380,etcd3=http://10.10.0.13:2380"
ETCD_INITIAL_CLUSTER_TOKEN="patroni-etcd-cluster"
ETCD_INITIAL_CLUSTER_STATE="new"

## sudo grep '^ETCD_' /etc/default/etcd

final clean up

sudo systemctl stop etcd

sudo systemctl reset-failed etcd

sudo find /var/lib/etcd -mindepth 1 -delete

sudo chown etcd:etcd /var/lib/etcd

sudo chmod 700 /var/lib/etcd

sudo find /var/lib/etcd -mindepth 1 -print

## Step 20 Start and verify etcd

Start etcd on all three nodes, then verify from any node.

sudo systemctl start --no-block etcd

after 10 seconds

sudo systemctl enable --now etcd
sudo systemctl status etcd --no-pager

export ETCDCTL_API=3
etcdctl --endpoints=http://10.10.0.11:2379,http://10.10.0.12:2379,http://10.10.0.13:2379 endpoint health
etcdctl --endpoints=http://10.10.0.11:2379 member list --write-out=table

# Phase 7 Configure Patroni

Use the same scope, etcd endpoints and passwords on all nodes. Change only the node name and node-specific listen addresses. Replace the example passwords before starting the cluster.

## Step 21 Create the node 1 configuration

sudo install -d -o postgres -g postgres -m 750 /etc/patroni
sudo nano /etc/patroni/patroni.yml

scope: postgres-ha
namespace: /service/
name: pg-node-1

restapi:
  listen: 10.10.0.11:8008
  connect_address: 10.10.0.11:8008

etcd3:
  hosts: 10.10.0.11:2379,10.10.0.12:2379,10.10.0.13:2379

bootstrap:
  dcs:
    ttl: 30
    loop_wait: 10
    retry_timeout: 10
    maximum_lag_on_failover: 1048576
    postgresql:
      use_pg_rewind: true
      use_slots: true
      parameters:
        wal_level: replica
        hot_standby: "on"
        max_wal_senders: 10
        max_replication_slots: 10
        wal_keep_size: 256MB
        password_encryption: scram-sha-256
  initdb:
    - encoding: UTF8
    - data-checksums
    - auth-local: peer
    - auth-host: scram-sha-256
  pg_hba:
    - local all all peer
    - host replication replicator 10.10.0.0/24 scram-sha-256
    - host all all 10.10.0.0/24 scram-sha-256
  users:
    training_admin:
      password: "TrainingAdmin-ChangeMe-2026"
      options:
        - createrole
        - createdb

postgresql:
  listen: 10.10.0.11:5432
  connect_address: 10.10.0.11:5432
  data_dir: /var/lib/postgresql/18/patroni
  bin_dir: /usr/lib/postgresql/18/bin
  pgpass: /var/lib/postgresql/.pgpass
  authentication:
    superuser:
      username: postgres
      password: "Postgres-ChangeMe-2026"
    replication:
      username: replicator
      password: "Replication-ChangeMe-2026"
    rewind:
      username: rewind_user
      password: "Rewind-ChangeMe-2026"
  parameters:
    unix_socket_directories: /var/run/postgresql
  create_replica_methods:
    - basebackup
  basebackup:
    checkpoint: fast

tags:
  nofailover: false
  noloadbalance: false
  clonefrom: false
  nosync: false

## Step 22 Adapt the file for pg-node-2

Use the complete node 1 file and replace only these values:

name: pg-node-2
restapi.listen: 10.10.0.12:8008
restapi.connect_address: 10.10.0.12:8008
postgresql.listen: 10.10.0.12:5432
postgresql.connect_address: 10.10.0.12:5432

## Step 23 Adapt the file for pg-node-3

name: pg-node-3
restapi.listen: 10.10.0.13:8008
restapi.connect_address: 10.10.0.13:8008
postgresql.listen: 10.10.0.13:5432
postgresql.connect_address: 10.10.0.13:5432

## Step 24 Secure and validate the YAML

sudo chown -R postgres:postgres /etc/patroni
sudo chmod 750 /etc/patroni
sudo chmod 600 /etc/patroni/patroni.yml
sudo -u postgres /opt/patroni/bin/patroni --validate-config /etc/patroni/patroni.yml

# Phase 8 Create the Patroni Service

Run on all three nodes.

sudo nano /etc/systemd/system/patroni.service

[Unit]
Description=Patroni PostgreSQL High Availability
After=network-online.target etcd.service
Wants=network-online.target
Requires=etcd.service

[Service]
Type=simple
User=postgres
Group=postgres
ExecStart=/opt/patroni/bin/patroni /etc/patroni/patroni.yml
ExecReload=/bin/kill -HUP $MAINPID
KillMode=process
TimeoutStopSec=30
Restart=on-failure
RestartSec=5

[Install]
WantedBy=multi-user.target

sudo systemctl daemon-reload
sudo systemctl enable patroni

# Phase 9 Bootstrap the Cluster

## Step 25 Start node 1 first

sudo systemctl start patroni
sudo journalctl -u patroni -n 100 --no-pager
sudo -u postgres /opt/patroni/bin/patronictl -c /etc/patroni/patroni.yml list

Node 1 should initially appear as Leader. Resolve errors before proceeding.

## Step 26 Start node 2

sudo systemctl start patroni
sudo -u postgres /opt/patroni/bin/patronictl -c /etc/patroni/patroni.yml list

## Step 27 Start node 3

sudo systemctl start patroni
sudo -u postgres /opt/patroni/bin/patronictl -c /etc/patroni/patroni.yml list

Expected result: one Leader and two streaming Replicas.

# Phase 10 Validate Replication

## Step 28 Create data on the leader

sudo -u postgres psql -c "CREATE DATABASE banking;"
sudo -u postgres psql -d banking -c "CREATE TABLE accounts(account_id bigint PRIMARY KEY, customer_name text NOT NULL, balance numeric(12,2) NOT NULL); INSERT INTO accounts VALUES (1001,'Anita',50000.00),(1002,'Bala',35000.00); SELECT * FROM accounts;"

## Step 29 Verify a replica

sudo -u postgres psql -d banking -c "SELECT * FROM accounts;"
sudo -u postgres psql -c "SELECT pg_is_in_recovery();"

pg_is_in_recovery() returns t on a replica and f on the leader.

## Step 30 Inspect replication from the leader

sudo -u postgres psql -x -c "SELECT application_name, client_addr, state, sync_state, sent_lsn, write_lsn, flush_lsn, replay_lsn FROM pg_stat_replication;"

# Before closing, optionally verify:

# sudo systemctl is-active patroni

# sudo systemctl is-active etcd

# Expected:

# active

# active

Later, reconnect to any node and check:

sudo -u postgres /opt/patroni/bin/patronictl \

-c /etc/patroni/patroni.yml list

# Phase 11 Test Automatic Failover

Identify the current leader with patronictl list.

On the leader, stop only Patroni: sudo systemctl stop patroni

On a surviving node, repeat patronictl list until a new leader is elected.

Insert a new row on the new leader.

Restart Patroni on the old leader and confirm that it rejoins as a replica.

sudo -u postgres /opt/patroni/bin/patronictl -c /etc/patroni/patroni.yml list
sudo systemctl stop patroni

sudo -u postgres psql -d banking -c "INSERT INTO accounts VALUES (1003,'Chitra',42000.00); SELECT * FROM accounts;"

sudo systemctl start patroni
sudo -u postgres /opt/patroni/bin/patronictl -c /etc/patroni/patroni.yml list

# Phase 12 Test Controlled Switchover

A switchover is planned movement of leadership to a healthy replica. Run from any healthy node:

sudo -u postgres /opt/patroni/bin/patronictl -c /etc/patroni/patroni.yml switchover

Choose the current leader, the candidate replica and now. Verify the new roles with patronictl list.

# Client Connectivity

Patroni does not provide a single stable application endpoint. During this lab, connect to the current leader private IP. A later exercise should add HAProxy or another Patroni-aware proxy. PgBouncer can then provide connection pooling behind or alongside that routing layer.

# Troubleshooting Reference

| Check | Command |
| --- | --- |
| Patroni service | sudo systemctl status patroni --no-pager |
| Patroni logs | sudo journalctl -u patroni -n 100 --no-pager |
| etcd service | sudo systemctl status etcd --no-pager |
| etcd logs | sudo journalctl -u etcd -n 100 --no-pager |
| PostgreSQL processes | ps -ef ¦ grep '[p]ostgres' |
| Listening ports | sudo ss -ltnp ¦ grep -E ':5432¦:8008¦:2379¦:2380' |
| Patroni state | curl http://10.10.0.11:8008/patroni |
| Leader endpoint | curl -i http://10.10.0.11:8008/primary |
| Database readiness | /usr/lib/postgresql/18/bin/pg_isready -h 127.0.0.1 -p 5432 |

# Stop and Cleanup

## Stop VMs and retain disks

gcloud compute instances stop pg-node-1 --zone=asia-south1-a
gcloud compute instances stop pg-node-2 --zone=asia-south1-b
gcloud compute instances stop pg-node-3 --zone=asia-south1-c

Stopped VMs do not incur vCPU and memory charges, but persistent disks and other retained resources can continue to incur charges.

## Permanent deletion order

Use these commands only when the lab is no longer required.

gcloud compute instances delete pg-node-1 --zone=asia-south1-a
gcloud compute instances delete pg-node-2 --zone=asia-south1-b
gcloud compute instances delete pg-node-3 --zone=asia-south1-c

gcloud compute routers nats delete postgres-lab-nat --router=postgres-lab-router --region=asia-south1
gcloud compute routers delete postgres-lab-router --region=asia-south1

gcloud compute firewall-rules delete patroni-cluster-internal patroni-iap-ssh
gcloud compute networks subnets delete postgres-lab-subnet --region=asia-south1
gcloud compute networks delete postgres-lab-vpc

# Completion Checklist

[ ] Correct GCP project selected and billing enabled

[ ] Budget alerts configured

[ ] VPC, subnet, IAP firewall rule and internal firewall rule created

[ ] Cloud NAT verified

[ ] Three private VMs reachable through IAP

[ ] Three healthy etcd members

[ ] One Patroni leader and two streaming replicas

[ ] Replication verified with test data

[ ] Automatic failover completed

[ ] Controlled switchover completed

[ ] Resources stopped or deleted after the lab